COB 200 White

USER MANUAL

Thank you for choosing to use our fixture. For the correct and safe use of this product, please read the instructions carefully before installing and using this product. This manual contains important installation and application information. When installing and operating products, please strictly follow the operation steps of the manual. At the same time, please keep this instruction properly.

DMX512 Connection

Please insert the head of XLR signal line equipped with this product into the signal output port of the controller and the mother into the signal input port of the rear panel of this product. You can connect multiple products in series. The signal line is three-core XLR input and output head with shielding layer wire. Please refer to the figure below.

For installations where the DMX cable must run a long distance or is in an electrically noisy environment, such as in a discotheque, it is recommended to use a DMX terminator. This helps in preventing corruption of the digital control signal by electrical noise. The DMX terminator is simply an XLR plug with a 120 Ω resistor connected between pins 2 and 3, which is then plugged into the output XLR socket of the last fixture in the chain. Please see illustrations below.

Screen settings

| Main Menu | Function set | Function description |
| --- | --- | --- |
| d001 | 000-512 | DMX-512 address set, 2 Channels mode |
| A001 | 000-512 | DMX-512 address set, 6 Channels mode |
| U255 | 000-255 | WW dimmer from dark to bright |
| C255 | 000-255 | CW dimmer from dark to bright |
| FH99 | 01-99 | Change Strobe flash speed |
| CL01 | 01-99 | Different colors output |
| CC99 | 01-99 | Colors jump change |
| DE99 | 01-99 | Colors gradate |
| CP99 | 01-99 | Colors pulse change |
| SU01 | 01-99 | Colors sound-active effects |

DMX charts

6 Channel Mode

| Channel | DMX Value | Function description |
| --- | --- | --- |
| 1 | 0-255 | Master dimmer from dark to bright |
| 2 | 0-255 | Warm white dimmer from dark to bright |
| 3 | 0-255 | Cold white dimmer from dark to bright |
| 4 | 0-255 | Strobe, from slow to fast |
| 5 |  | Function selection |
|  | 0-50 | DMX control |
|  | 51-100 | Different colors |
|  | 101-150 | Jumping change |
|  | 151-200 | Gradually change |
|  | 201-250 | Pulse change |
|  | 251-255 | Sound control |

2 Channel Mode

| Channel | DMX Value | Function description |
| --- | --- | --- |
| 1 | 0-255 | Warm white dimmer from dark to bright |
| 2 | 0-255 | Cold white dimmer from dark to bright |

Technical parameters

| Source |  |
| --- | --- |
| LED | 200W Chip-on-board WW+CW 2-in-1 |
| Color temperature | Adjustable 3200K ~ 5700K |
| Lifespan | 50 000 hours |
| Optical system |  |
| Beam angle | 60° |
| Functions |  |
| CTO | Linear |
| Strobe | Electrical 1-20 fps |
| Dimer | Electrical 0-100% |
| Control |  |
| Multifunctional LCD display |  |
| DMX channels | 2/6 |
| Modes | DMX512/master-slave/auto/sound activation |
| Power supply |  |
| Input voltage | AC 90-240V ,50/60 Hz |
| Power consumption | 200W |
| Connections |  |
| DMX512 | XLR 3-pin in/out |
| Power | Build-in |
| Package contains |  |
| Fixture | 1 pc |
| User manual | 1 pc |
| Dimentions |  |
| Fixture | 225x300x230 mm |
| Carton | 230x230x350 mm |
| Weight |  |
| N.W. | 3 kg |
| G.W. (carton) | 4 kg |